SECRET

FUNCTION INTRODUCTION

| Function management |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Function Name: |  | Hill Descent Control |  |  |  |
| Function Mapping: |  | Driver assistance, autonomous driving, and chassis management |  |  |  |
| Function: |  |  |  | Function-ID: |  |
|  | Name |  | Roler | Date | signature |
| Create/Modify: |  |  |  |  |  |
| check: |  |  |  |  |  |
| Approve OMK: |  |  |  |  |  |

1	Function Introduction	3

1.1	Function description	3

1.2	Function partner list	3

1.3	Function action mode diagram	4

2	System Architecture	4

2.1	Function architecture	4

2.2	Sub-function list	5

2.3	Description of function interface	5

3	Basis function specification	6

3.1	Description of the function status	6

3.2	Logical signals of function	7

3.3	Base Parameters of function	8

3.4	State interpretation and diagram	9

3.5	Diagnostic requirements	9

4	Major Additional Function	10

5	Special States Detect	10

6	Functional safety	10

6.1	Hazard and Risk analysis	10

6.2	Functional safety concept	10

7	Regulatory Requirements	11

8	Function Dependencies	11

8.1	Internal	11

8.2	Extra	11

9	Special Performance Requirements	12

10	Appendix A	12

10.1	Change History	12

# Function Introduction

## Function description

HDC (Hill Descent Control) autonomously controls the brake torques to maintain a customer set target speed. The feature is commonly referred to as Hill Descent Control by a majority of OEMs. When the feature is actively controlling, the driver has the ability to override the HDC feature with the use of the brake or accelerator pedal and the transitions should be seamless to the driver.

At the vehicle level, this feature is a low speed (steep) downhill cruise control feature typically intended for use on unmaintained surfaces (off-road).

Figure1. Hill Descent Control Demonstration

## Function partner list

- iBRS

- Wheel Speed Sensor

- IMU

- LDCU

- HMI

## Function action mode diagram

The function 'Hill Descent Control' calculates a request for the hydraulic brake system to build up hydraulic brake force to decelerate the vehicle when function has been activated by the driver.

The diagrams are based on ZF IBC (Integrated Brake Controller) system.

Figure2. Hydraulic Schematic

# System Architecture

## Function architecture

The HDC function is integrated in the braking product, receiving information from the four wheel speed sensors, HMI, and IMU, and calculating and outputting the target pressure at the four wheel ends.

Figure 5. HDC system architecture

## Sub-function list

N/A

## Description of function interface

### System dependency (Signals Interaction)

The following table lists the interactive signals for HDC function. The names in the table are from VW CMP21D CEA Comms matrix “IBRS_CEA2_CAN_MATRIX”.

| Signal Name | Sender | Tx/Rx | Cycle | Unit | Range | ASIL Level | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LDCU_EVSysReadySt | LDCU | Rx | 10ms | Enum | 0x0:Not ready; 0x1:High Voltage On; 0x2:Ready; | ASIL B | The state of vehicle |
| LDCU_CurrentGearLev | LDCU | Rx | 10ms | Enum | 0x0:Invalid; 0x1:D; 0x2:N; 0x3:R; 0x4:P; 0x5-0x7:Reserved | ASIL B | The Current Gear |
| LDCU_AccPedalSig | LDCU | Rx | 10ms | % | 0-100 | ASIL C | Accelerator pedal position signal |
| LDCU_AccPedalSigVD | LDCU | Rx | 10ms | Boolean | 0-1 | ASIL C | The valid of accelerator pedal position signal |
| CDCU_ESC_HDCSW | CDCU | Rx | 100ms | Enum | 0x0:No command 0x1:On 0x2:Off 0x3:Reserved | ASIL C | HDC SW switch |
| CDCU_ESC_PATASw | CDCU | Rx | 100ms | Enum | 0x0:No command; 0x1:Normal; 0x2:Sport; 0x3:Off; | QM | ESC_OFF SW |
| LongAcceleration | IMU | Rx(Private) | 10ms | g | -2~2 | ASIL B (D) | Longitudinal Acceleration |
| LongAccelerationValid | IMU | Rx(Private) | 10ms | Enum | 0x0:valid 0x1:invalid 0x2:reserved 0x3:initialization | ASIL B (D) | The valid of longitudinal acceleration |
| IPB_HDCFault | iBRS | Tx | 20ms | Enum | 0x0:Not fault; 0x1:fault | ASIL B | HDC fault state |
| IPB_HDCActiveSt | iBRS | Tx | 20ms | Enum | 0x0:Not Active; 0x1:standby 0x2:Active; 0x3:reserved | ASIL B | HDC activation status |
| IPB_CruiseCtrlCancelReq | iBRS | Tx | 10ms | Enum | 0x0:No request; 0x1:Request; | ASIL B | ESP request LDCU to cancel cruise control |
| IPB_BrkLightOnReq | iBRS | Tx | 20ms | Boolean | 0-1 | ASIL B | Brake Light On Request |

### Hardware Interface

| Name | Type |
| --- | --- |
| KL15 | Hardwired |
| KL30 | Hardwired |
| Left front wheel speed sensor | intelligent |
| Right front wheel speed sensor | intelligent |
| Left rear wheel speed sensor | intelligent |
| Right rear wheel speed sensor | intelligent |
| Master Cylinder Pressure sensor | intelligent |
| IPB CAN(H、L) | Hardwired |

# Basis function specification

## Description of the function status

The HDC function activate when the vehicle is on the downhill and the driver activate the function via HMI, then the driver can release the brake and let the HDC function to control the deceleration during downhill, while the driver can adjust the HDC target speed by pressing brake pedal or accelerator pedal.

The fig.6 introduce the typical HDC working process:

a. HDC SW switch on HDC status will transit from No Active to standby

b. In case of activation condition fulfilled, HDC will transit from standby to Active

c. In case of activation conditions not fulfilled, HDC will transit from Active to standby

d. In case of vehicle speed below HDC target speed, HDC will transit from Active to standby

e. In case of brake disc overheat, HDC will transit from Active to No active

f. In case of HDC SW switch off, HDC will transit from Active to No active

Figure 6. HDC Schematic Diagram Of The Work

## Logical signals of function

The conditions of HDC enable and activation should meet the following conditions:

- The enable conditions of HDC function:

- The feature is not degraded by fault

And

- Driver activate HDC SW switch

And

- The system state is in running

- The disable conditions of HDC function:

- The feature is degraded by fault

OR

- Driver deactivate HDC SW switch

OR

-  The system state is in not running

OR

- Brake Disc Temperature > TBD7

OR

- Accelerator pedal > TBD8

OR

- Vehicle speed is higher than 60kph

- The activation conditions of HDC function:

- Vehicle speed is lower than 35kph

And

- Hill Slope > 4%

And

- The vehicle speed is higher than HDC target speed

And

- The vehicle not in P gear position

And

- Brake Disc temperature < TBD1

And

- Master Cylinder Pressure < TBD2

And

- Accelerator pedal < TBD3

- The deactivation conditions of HDC function:

- Vehicle speed is lower than TBD4 (stop)

OR

- Hill Slope < 4%

OR

- The vehicle speed is lower than HDC target speed

OR

- Master Cylinder Pressure > TBD5

OR

- Accelerator pedal > TBD6

## Base Parameters of function

This table defines the required threshold parameters in the function logics. These are all base values

for a specific platform or vehicle. The thresholds are available to be tuned and verified on the test

vehicles during development.

| Conditions |  | Threshold | Remark |
| --- | --- | --- | --- |
| Activation Condition | Vehicle Speed | < 35 (kph) | Tunable |
|  | Hill Slope | > 4 (%) | Tunable |
|  | Brake Disc Temp | < TBD1 (℃) | Tunable |
|  | Master Cylinder Pressure | < TBD2 (Bar) | Tunable |
|  | Accelerator Pedal | < TBD3 (%) | Tunable |
| Deactivation Condition | Vehicle Speed | < TBD4 (kph) | Tunable |
|  | Hill Slope | < 4 (%) | Tunable |
|  | Master Cylinder Pressure | > TBD5 (Bar) | Tunable |
|  | Accelerator Pedal | > TBD6 (%) | Tunable |
| Disable condition | Brake Disc Temp | > TBD7 (℃) | Tunable |
|  | Accelerator Pedal | > TBD6 (%) | Tunable |
|  | Vehicle Speed | > 60 (kph) | Tunable |

For a best performance expectation, the software is also required to support other variables change

Interface.

## State interpretation and diagram

Several state groups are defined in HDC control, including the following main groups:

- Not Active

- standby

- Active

The picture shows the HDC subsystem work flow diagram.

## Diagnostic requirements

When there is a system malfunction that affects the ability for HDC to run normal control, then the fault manager will request that HDC enter a special control mode.

### Special Control Modes

- CM_LC_HDC_NORMAL

- CM_LC_HDC_INHIBIT

### Partial System Shutdown Modes

If a failure occurs while HDC is already active then will enter a Partial System Shutdown (PSSD) mode.  It depends on the particular fault and the current status of the control, but a given channel could be requested to run one of the following modes:

- PSSD_LC_HDC_NORMAL

- PSSD_LC_HDC_RAMP_DWN_SLW

# Major Additional Function

NA

# Special States Detect

NA

# Functional safety

## Hazard and Risk analysis

This part will be detailed in HARA. Refer to the HARA documents provided by VW.

## Functional safety concept

This part will be detailed in FSC. Refer to the FSC documents provided by VW.

### Degradation Mode

Refer to the failsafe manual, which defines details and cases. The failsafe manual should be updated during each software release

# Regulatory Requirements

NA

# Function Dependencies

| External systems | Description | Responsible |
| --- | --- | --- |
| LDCU | Vehicle Info | LDCU |
| IMU | Acceleration and Yaw Single | EK |
| EPS | Steering ability | EF |
| Foundation Braking | Calipers, discs, tubing | EF |

## Internal

### Arbitration of HDC and ADAS

HDC has higher priority than ADAS deceleration control.

In the case of HDC Active, ADAS cannot be active, via interface of IPB_CruiseCtrlCancelReq;

In the case of ADAS Active, HDC switch can be turn on and HDC enter standby status, if HDC

activation condition fulfilled HDC Active, then ADAS shall be disable via interface of

IPB_CruiseCtrlCancelReq;

## Extra

### HDC Switch：

HDC function can be switched off and on via the virtual switch on the screen by the driver. The

request signal transmit to iBRS from the CDCU.

## 8.2.1 Self-checking strategy

After KL15 on, the system will send the default value until self check complete. The HMI will illuminate

the HDC lamp and keep on 3s.

## 8.2.2 Function Active Display Strategy

The HMI strategy when HDC function active：

| Name | Symbol | KL15 Status | HMI Strategy |
| --- | --- | --- | --- |
| HDC Active |  | ON | 仪表显示策略： 1. 仪表如果收到 IPB_HDCActiveSt = 0x0:NotActive / 0x1：standby / 0x3:reserved 且 IPB_HDCFault = 0x0:Notfault，不点亮指示灯； 2. 仪表如果收到IPB_HDCActiveSt==Standby且IPB_HDCFault==Notfault，点亮灰色指 示灯； 3. 仪表如果收到 IPB_HDCActiveSt = 0x2:Active 且 IPB_HDCFault = 0x0:Not fault，点亮 绿色指示灯，并弹出文字提示“陡坡缓降功能已激活”，3s 后文字消失（无论是否有 效）。直到下一次检测到IPB_HDCActiveSt 从其他状态变为0x2:Active 且 IPB_HDCFault = 0x0： Not fault； 异常处理： 如果收不到 IPB_HDCActiveSt 或 IPB_HDCFault 信号 1s 按通用策略 10 帧认为丢失，点亮黄色指示灯。 备注： N/A |

When the HDC activate, need to illuminate the brake lamp.

## 8.2.3 Fault Display Strategy

| Name | Symbol | KL15 Status | HMI Strategy |
| --- | --- | --- | --- |
| HDC Fault |  | ON | 仪表显示策略： 仪表如果收到 IPB_HDCFault = 0x1:Fault，点亮黄色指示； 大屏显示策略： 大屏收到HDC故障信号：IPB_HDCFault=0x1:Fault，大屏显示文字标题：“陡坡缓降功能不可用” 文 字内容“陡坡缓降功能不可用，无法辅助车辆保持匀速下坡” 异常处理： 如果收不到 IPB_HDCActiveSt 或 IPB_HDCFault 信号 1s 按通用策略 10 帧认为丢失，点亮黄色指示灯。 备注： NA |

# Special Performance Requirements

NA.

# Appendix A

## Change History

| Version | Change Content | Date（D/M/Y） | Reviewer |
| --- | --- | --- | --- |
| V1.0 | Initial edition | 7/07/2025 | Chen Haiyan |